МИНИСТЕРСТВО НАУКИ И ВЫСШЕГО ОБРАЗОВАНИЯ

РОССИЙСКОЙ ФЕДЕРАЦИИ

федеральное государственное бюджетное образовательное учреждение

высшего образования

«Курганский государственный университет»

Кафедра «Программное обеспечение автоматизированных систем»

Проектирование локальной вычислительной сети

Конфигурация оборудования лвс

РФ КГУ 09.03.03КП25 260091 К1

Листов 15

Курган 2025

Для пользовательского сегмента используются 12 коммутаторов уровня доступа, для серверного – 3. Ниже приведено по одной конфигурации для коммутатора из серверного сегмента и коммутатора из пользовательского сегмента соответственно. У остальных коммутаторов доступа те же самые настройки, отличаются только настройки портов, в которые подключаются хосты (разные Vlan’ы).

Листинги настроек конфигурации сокращены для всех устройств в местах, где это допустимо (нет настроек).

Листинг 1. Running-Config коммутатора SRV1

Current configuration : 1508 bytes

!

version 15.0

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname Server101-SW

!

ip ssh version 1

ip domain-name organisation.local

!

username admin secret 5 $1$mERr$RldxcCZEZsTFTETUyRaA50

!

spanning-tree mode pvst

spanning-tree extend system-id

!

interface FastEthernet0/1

switchport access vlan 101

switchport mode access

spanning-tree portfast

!

interface FastEthernet0/2

switchport trunk allowed vlan 10,20,30,101-102

switchport mode trunk

!

interface FastEthernet0/3

switchport trunk allowed vlan 10,20,30,101-102

switchport mode trunk

!

interface Vlan1

no ip address

shutdown

!

snmp-server community organisationX_RO RO

snmp-server community organisationX_RW RW

!

line con 0

!

line vty 0 4

login local

line vty 5 15

login local

!

end

Листинг 2. Running-Config коммутатора пользовательского сегмента SEG1-48:

Current configuration : 1506 bytes

!

version 15.0

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname Section1-48

!

ip ssh version 1

ip domain-name organisation.local

!

username admin secret 5 $1$mERr$RldxcCZEZsTFTETUyRaA50

!

spanning-tree mode pvst

spanning-tree extend system-id

!

interface FastEthernet0/1

switchport access vlan 10

switchport mode access

spanning-tree portfast

!

interface FastEthernet0/2

switchport trunk allowed vlan 10,20,30,101-102

switchport mode trunk

!

interface FastEthernet0/3

switchport trunk allowed vlan 10,20,30,101-102

switchport mode trunk

!

interface Vlan1

no ip address

shutdown

!

snmp-server community organisationX_RO RO

snmp-server community organisationX_RW RW

!

line con 0

!

line vty 0 4

login local

line vty 5 15

login local

!

end

Листинг 3. Running-Config коммутатора MSW1

Current configuration : 5394 bytes

!

version 12.2(37)SE1

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname MSW1

!

ip dhcp excluded-address 10.0.0.1

ip dhcp excluded-address 10.0.0.2

ip dhcp excluded-address 10.0.0.3

ip dhcp excluded-address 10.0.0.65

ip dhcp excluded-address 10.0.0.66

ip dhcp excluded-address 10.0.0.67

ip dhcp excluded-address 10.0.0.129

ip dhcp excluded-address 10.0.0.130

ip dhcp excluded-address 10.0.0.131

ip dhcp excluded-address 10.0.1.129

ip dhcp excluded-address 10.0.1.130

ip dhcp excluded-address 10.0.1.131

ip dhcp excluded-address 10.0.1.161

ip dhcp excluded-address 10.0.1.162

ip dhcp excluded-address 10.0.1.163

!

ip dhcp pool VLAN10_MAIN

network 10.0.0.0 255.255.255.192

default-router 10.0.0.1

dns-server 10.0.100.34

ip dhcp pool VLAN20_MAIN

network 10.0.0.64 255.255.255.192

default-router 10.0.0.65

dns-server 10.0.100.34

ip dhcp pool VLAN30_MAIN

network 10.0.0.128 255.255.255.192

default-router 10.0.0.129

dns-server 10.0.100.34

ip dhcp pool VLAN101_MAIN

network 10.0.1.128 255.255.255.224

default-router 10.0.1.129

dns-server 10.0.100.34

ip dhcp pool VLAN102_MAIN

network 10.0.1.160 255.255.255.224

default-router 10.0.1.161

dns-server 10.0.100.34

!

ip routing

!

username admin privilege 15 secret 5 $1$mERr$RldxcCZEZsTFTETUyRaA50

!

ip ssh version 1

ip domain-name organisation.local

!

spanning-tree mode pvst

spanning-tree vlan 10,20,30 priority 24576

spanning-tree vlan 101-102 priority 28672

!

interface Port-channel12

switchport trunk allowed vlan 10,20,30,101-102

switchport trunk encapsulation dot1q

switchport mode trunk

!

interface Port-channel13

no switchport

ip address 10.0.100.1 255.255.255.252

!

interface FastEthernet0/1

switchport trunk allowed vlan 10,20,30,101-102

switchport trunk encapsulation dot1q

switchport mode trunk

!

interface FastEthernet0/2

switchport trunk allowed vlan 10,20,30,101-102

switchport trunk encapsulation dot1q

switchport mode trunk

!

interface FastEthernet0/3

switchport trunk allowed vlan 10,20,30,101-102

switchport trunk encapsulation dot1q

switchport mode trunk

!

interface FastEthernet0/4

switchport trunk allowed vlan 10,20,30,101-102

switchport trunk encapsulation dot1q

switchport mode trunk

!

interface FastEthernet0/5

switchport trunk allowed vlan 10,20,30,101-102

switchport trunk encapsulation dot1q

switchport mode trunk

!

interface FastEthernet0/6

switchport trunk allowed vlan 10,20,30,101-102

switchport trunk encapsulation dot1q

switchport mode trunk

!

interface FastEthernet0/7

switchport trunk allowed vlan 10,20,30,101-102

switchport trunk encapsulation dot1q

switchport mode trunk

!

interface FastEthernet0/8

switchport trunk allowed vlan 10,20,30,101-102

switchport trunk encapsulation dot1q

switchport mode trunk

!

interface FastEthernet0/21

no switchport

ip address 10.0.100.9 255.255.255.252

duplex auto

speed auto

!

interface FastEthernet0/22

no switchport

ip address 10.0.100.13 255.255.255.252

duplex auto

speed auto

!

interface FastEthernet0/23

no switchport

no ip address

channel-group 13 mode active

duplex auto

speed auto

!

interface FastEthernet0/24

no switchport

no ip address

channel-group 13 mode active

duplex auto

speed auto

!

interface GigabitEthernet0/1

switchport trunk allowed vlan 10,20,30,101-102

switchport trunk encapsulation dot1q

switchport mode trunk

channel-group 12 mode active

!

interface GigabitEthernet0/2

switchport trunk allowed vlan 10,20,30,101-102

switchport trunk encapsulation dot1q

switchport mode trunk

channel-group 12 mode active

!

interface Vlan1

no ip address

shutdown

!

interface Vlan10

mac-address 00e0.a324.a201

ip address 10.0.0.2 255.255.255.192

standby 10 ip 10.0.0.1

standby 10 priority 120

standby 10 preempt

!

interface Vlan20

mac-address 00e0.a324.a202

ip address 10.0.0.66 255.255.255.192

standby 20 ip 10.0.0.65

standby 20 priority 120

standby 20 preempt

!

interface Vlan30

mac-address 00e0.a324.a203

ip address 10.0.0.130 255.255.255.192

standby 30 ip 10.0.0.129

standby 30 priority 120

standby 30 preempt

!

interface Vlan101

mac-address 00e0.a324.a204

ip address 10.0.1.130 255.255.255.224

standby 101 ip 10.0.1.129

standby 101 priority 120

standby 101 preempt

!

interface Vlan102

mac-address 00e0.a324.a205

ip address 10.0.1.162 255.255.255.224

standby 102 ip 10.0.1.161

standby 102 priority 120

standby 102 preempt

!

router ospf 1

router-id 1.1.1.1

log-adjacency-changes

network 10.0.0.0 0.0.0.63 area 0

network 10.0.0.64 0.0.0.63 area 0

network 10.0.0.128 0.0.0.63 area 0

network 10.0.1.128 0.0.0.31 area 0

network 10.0.1.160 0.0.0.31 area 0

network 10.0.100.0 0.0.0.3 area 0

network 10.0.100.8 0.0.0.3 area 0

network 10.0.100.12 0.0.0.3 area 0

!

ip classless

!

ip flow-export version 9

!

no cdp run

!

snmp-server community organisationX_RO RO

snmp-server community organisationX_RW RW

!

line con 0

!

line aux 0

!

line vty 0 4

login local

line vty 5 15

login local

!

end

Листинг 4. Running-Config коммутатора MSW2

Current configuration : 5047 bytes

!

version 12.2(37)SE1

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname MSW2

!

ip dhcp excluded-address 10.0.0.1

ip dhcp excluded-address 10.0.0.2

ip dhcp excluded-address 10.0.0.3

ip dhcp excluded-address 10.0.0.65

ip dhcp excluded-address 10.0.0.66

ip dhcp excluded-address 10.0.0.67

ip dhcp excluded-address 10.0.0.129

ip dhcp excluded-address 10.0.0.130

ip dhcp excluded-address 10.0.0.131

ip dhcp excluded-address 10.0.1.129

ip dhcp excluded-address 10.0.1.130

ip dhcp excluded-address 10.0.1.131

ip dhcp excluded-address 10.0.1.161

ip dhcp excluded-address 10.0.1.162

ip dhcp excluded-address 10.0.1.163

!

ip dhcp pool VLAN10_BACKUP

network 10.0.0.0 255.255.255.192

default-router 10.0.0.1

dns-server 10.0.100.34

ip dhcp pool VLAN20_BACKUP

network 10.0.0.64 255.255.255.192

default-router 10.0.0.65

dns-server 10.0.100.34

ip dhcp pool VLAN30_BACKUP

network 10.0.0.128 255.255.255.192

default-router 10.0.0.129

dns-server 10.0.100.34

ip dhcp pool VLAN101_BACKUP

network 10.0.1.128 255.255.255.224

default-router 10.0.1.129

dns-server 10.0.100.34

ip dhcp pool VLAN102_BACKUP

network 10.0.1.160 255.255.255.224

default-router 10.0.1.161

dns-server 10.0.100.34

!

!

ip routing

!

username admin privilege 15 secret 5 $1$mERr$RldxcCZEZsTFTETUyRaA50

!

ip ssh version 1

ip domain-name organisation.local

!

spanning-tree mode pvst

spanning-tree vlan 101-102 priority 24576

spanning-tree vlan 10,20,30 priority 28672

!

interface Port-channel12

switchport trunk allowed vlan 10,20,30,101-102

switchport trunk encapsulation dot1q

switchport mode trunk

!

interface Port-channel24

no switchport

ip address 10.0.100.5 255.255.255.252

!

interface FastEthernet0/1

switchport trunk allowed vlan 10,20,30,101-102

switchport trunk encapsulation dot1q

switchport mode trunk

!

interface FastEthernet0/2

switchport trunk allowed vlan 10,20,30,101-102

switchport trunk encapsulation dot1q

switchport mode trunk

!

interface FastEthernet0/3

switchport trunk allowed vlan 10,20,30,101-102

switchport trunk encapsulation dot1q

switchport mode trunk

!

interface FastEthernet0/4

switchport trunk allowed vlan 10,20,30,101-102

switchport trunk encapsulation dot1q

switchport mode trunk

!

interface FastEthernet0/5

switchport trunk allowed vlan 10,20,30,101-102

switchport trunk encapsulation dot1q

switchport mode trunk

!

interface FastEthernet0/6

switchport trunk allowed vlan 10,20,30,101-102

switchport trunk encapsulation dot1q

switchport mode trunk

!

interface FastEthernet0/7

switchport trunk allowed vlan 10,20,30,101-102

switchport trunk encapsulation dot1q

switchport mode trunk

!

interface FastEthernet0/8

switchport trunk allowed vlan 10,20,30,101-102

switchport trunk encapsulation dot1q

switchport mode trunk

!

interface FastEthernet0/23

no switchport

no ip address

channel-group 24 mode active

duplex auto

speed auto

!

interface FastEthernet0/24

no switchport

no ip address

channel-group 24 mode active

duplex auto

speed auto

!

interface GigabitEthernet0/1

switchport trunk allowed vlan 10,20,30,101-102

switchport trunk encapsulation dot1q

switchport mode trunk

channel-group 12 mode active

!

interface GigabitEthernet0/2

switchport trunk allowed vlan 10,20,30,101-102

switchport trunk encapsulation dot1q

switchport mode trunk

channel-group 12 mode active

!

interface Vlan1

no ip address

shutdown

!

interface Vlan10

mac-address 000a.f3a6.5201

ip address 10.0.0.3 255.255.255.192

standby 10 ip 10.0.0.1

standby 10 preempt

!

interface Vlan20

mac-address 000a.f3a6.5202

ip address 10.0.0.67 255.255.255.192

standby 20 ip 10.0.0.65

standby 20 preempt

!

interface Vlan30

mac-address 000a.f3a6.5203

ip address 10.0.0.131 255.255.255.192

standby 30 ip 10.0.0.129

standby 30 preempt

!

interface Vlan101

mac-address 000a.f3a6.5204

ip address 10.0.1.131 255.255.255.224

standby 101 ip 10.0.1.129

standby 101 preempt

!

interface Vlan102

mac-address 000a.f3a6.5205

ip address 10.0.1.163 255.255.255.224

standby 102 ip 10.0.1.161

standby 102 preempt

!

router ospf 1

router-id 1.1.1.2

log-adjacency-changes

network 10.0.0.0 0.0.0.63 area 0

network 10.0.0.64 0.0.0.63 area 0

network 10.0.0.128 0.0.0.63 area 0

network 10.0.1.128 0.0.0.31 area 0

network 10.0.1.160 0.0.0.31 area 0

network 10.0.100.4 0.0.0.3 area 0

!

ip classless

!

ip flow-export version 9

!

no cdp run

!

snmp-server community organisationX_RO RO

snmp-server community organisationX_RW RW

!

line con 0

!

line aux 0

!

line vty 0 4

login local

line vty 5 15

login local

!

end

Листинг 5. Running-Config коммутатора MSW3

Current configuration : 4825 bytes

!

version 12.2(37)SE1

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname MSW3

!

!

!

ip dhcp excluded-address 10.0.0.193

ip dhcp excluded-address 10.0.0.194

ip dhcp excluded-address 10.0.0.195

ip dhcp excluded-address 10.0.1.1

ip dhcp excluded-address 10.0.1.2

ip dhcp excluded-address 10.0.1.3

ip dhcp excluded-address 10.0.1.65

ip dhcp excluded-address 10.0.1.66

ip dhcp excluded-address 10.0.1.67

ip dhcp excluded-address 10.0.1.193

ip dhcp excluded-address 10.0.1.194

ip dhcp excluded-address 10.0.1.195

!

ip dhcp pool VLAN40_MAIN

network 10.0.0.192 255.255.255.192

default-router 10.0.0.193

dns-server 10.0.100.34

ip dhcp pool VLAN50_MAIN

network 10.0.1.0 255.255.255.192

default-router 10.0.1.1

dns-server 10.0.100.34

ip dhcp pool VLAN60_MAIN

network 10.0.1.64 255.255.255.192

default-router 10.0.1.65

dns-server 10.0.100.34

ip dhcp pool VLAN103_MAIN

network 10.0.1.192 255.255.255.224

default-router 10.0.1.193

dns-server 10.0.100.34

!

ip routing

!

username admin privilege 15 secret 5 $1$mERr$RldxcCZEZsTFTETUyRaA50

!

ip ssh version 1

ip domain-name organisation.local

!

spanning-tree mode pvst

spanning-tree vlan 40,50 priority 24576

spanning-tree vlan 60,103 priority 28672

!

interface Port-channel13

no switchport

ip address 10.0.100.2 255.255.255.252

!

interface Port-channel34

switchport trunk allowed vlan 40,50,60,103

switchport trunk encapsulation dot1q

switchport mode trunk

!

interface FastEthernet0/1

switchport trunk allowed vlan 40,50,60,103

switchport trunk encapsulation dot1q

switchport mode trunk

!

interface FastEthernet0/2

switchport trunk allowed vlan 40,50,60,103

switchport trunk encapsulation dot1q

switchport mode trunk

!

interface FastEthernet0/3

switchport trunk allowed vlan 40,50,60,103

switchport trunk encapsulation dot1q

switchport mode trunk

!

interface FastEthernet0/4

switchport trunk allowed vlan 40,50,60,103

switchport trunk encapsulation dot1q

switchport mode trunk

!

interface FastEthernet0/5

switchport trunk allowed vlan 40,50,60,103

switchport trunk encapsulation dot1q

switchport mode trunk

!

interface FastEthernet0/6

switchport trunk allowed vlan 40,50,60,103

switchport trunk encapsulation dot1q

switchport mode trunk

!

interface FastEthernet0/7

switchport trunk allowed vlan 40,50,60,103

switchport trunk encapsulation dot1q

switchport mode trunk

!

interface FastEthernet0/21

no switchport

ip address 10.0.100.21 255.255.255.252

duplex auto

speed auto

!

interface FastEthernet0/22

no switchport

ip address 10.0.100.17 255.255.255.252

duplex auto

speed auto

!

interface FastEthernet0/23

no switchport

no ip address

channel-group 13 mode active

duplex auto

speed auto

!

interface FastEthernet0/24

no switchport

no ip address

channel-group 13 mode active

duplex auto

speed auto

!

interface GigabitEthernet0/1

switchport trunk allowed vlan 40,50,60,103

switchport trunk encapsulation dot1q

switchport mode trunk

channel-group 34 mode active

!

interface GigabitEthernet0/2

switchport trunk allowed vlan 40,50,60,103

switchport trunk encapsulation dot1q

switchport mode trunk

channel-group 34 mode active

!

interface Vlan1

no ip address

shutdown

!

interface Vlan40

mac-address 0060.4719.cb01

ip address 10.0.0.194 255.255.255.192

standby 40 ip 10.0.0.193

standby 40 priority 120

standby 40 preempt

!

interface Vlan50

mac-address 0060.4719.cb02

ip address 10.0.1.2 255.255.255.192

standby 50 ip 10.0.1.1

standby 50 priority 120

standby 50 preempt

!

interface Vlan60

mac-address 0060.4719.cb03

ip address 10.0.1.66 255.255.255.192

standby 60 ip 10.0.1.65

standby 60 priority 120

standby 60 preempt

!

interface Vlan103

mac-address 0060.4719.cb04

ip address 10.0.1.194 255.255.255.224

standby 103 ip 10.0.1.193

standby 103 priority 120

standby 103 preempt

!

router ospf 1

router-id 1.1.2.1

log-adjacency-changes

network 10.0.0.192 0.0.0.63 area 0

network 10.0.1.0 0.0.0.63 area 0

network 10.0.1.64 0.0.0.63 area 0

network 10.0.1.192 0.0.0.31 area 0

network 10.0.100.0 0.0.0.3 area 0

network 10.0.100.16 0.0.0.3 area 0

network 10.0.100.20 0.0.0.3 area 0

!

ip classless

!

ip flow-export version 9

!

no cdp run

!

snmp-server community organisationX_RO RO

snmp-server community organisationX_RW RW

!

line con 0

!

line aux 0

!

line vty 0 4

login local

line vty 5 15

login local

!

end

Листинг 6. Running-Config коммутатора MSW4

Current configuration : 4499 bytes

!

version 12.2(37)SE1

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname MSW4

!

ip dhcp excluded-address 10.0.0.193

ip dhcp excluded-address 10.0.0.194

ip dhcp excluded-address 10.0.0.195

ip dhcp excluded-address 10.0.1.1

ip dhcp excluded-address 10.0.1.2

ip dhcp excluded-address 10.0.1.3

ip dhcp excluded-address 10.0.1.65

ip dhcp excluded-address 10.0.1.66

ip dhcp excluded-address 10.0.1.67

ip dhcp excluded-address 10.0.1.193

ip dhcp excluded-address 10.0.1.194

ip dhcp excluded-address 10.0.1.195

!

ip dhcp pool VLAN40_BACKUP

network 10.0.0.192 255.255.255.192

default-router 10.0.0.193

dns-server 10.0.100.34

ip dhcp pool VLAN50_BACKUP

network 10.0.1.0 255.255.255.192

default-router 10.0.1.1

dns-server 10.0.100.34

ip dhcp pool VLAN60_BACKUP

network 10.0.1.64 255.255.255.192

default-router 10.0.1.65

dns-server 10.0.100.34

ip dhcp pool VLAN103_BACKUP

network 10.0.1.192 255.255.255.224

default-router 10.0.1.193

dns-server 10.0.100.34

!

ip routing

!

username admin privilege 15 secret 5 $1$mERr$RldxcCZEZsTFTETUyRaA50

!

ip ssh version 1

ip domain-name organisation.local

!

spanning-tree mode pvst

spanning-tree vlan 60,103 priority 24576

spanning-tree vlan 40,50 priority 28672

!

interface Port-channel24

no switchport

ip address 10.0.100.6 255.255.255.252

!

interface Port-channel34

switchport trunk allowed vlan 40,50,60,103

switchport trunk encapsulation dot1q

switchport mode trunk

!

interface FastEthernet0/1

switchport trunk allowed vlan 40,50,60,103

switchport trunk encapsulation dot1q

switchport mode trunk

!

interface FastEthernet0/2

switchport trunk allowed vlan 40,50,60,103

switchport trunk encapsulation dot1q

switchport mode trunk

!

interface FastEthernet0/3

switchport trunk allowed vlan 40,50,60,103

switchport trunk encapsulation dot1q

switchport mode trunk

!

interface FastEthernet0/4

switchport trunk allowed vlan 40,50,60,103

switchport trunk encapsulation dot1q

switchport mode trunk

!

interface FastEthernet0/5

switchport trunk allowed vlan 40,50,60,103

switchport trunk encapsulation dot1q

switchport mode trunk

!

interface FastEthernet0/6

switchport trunk allowed vlan 40,50,60,103

switchport trunk encapsulation dot1q

switchport mode trunk

!

interface FastEthernet0/7

switchport trunk allowed vlan 40,50,60,103

switchport trunk encapsulation dot1q

switchport mode trunk

!

interface FastEthernet0/23

no switchport

no ip address

channel-group 24 mode active

duplex auto

speed auto

!

interface FastEthernet0/24

no switchport

no ip address

channel-group 24 mode active

duplex auto

speed auto

!

interface GigabitEthernet0/1

switchport trunk allowed vlan 40,50,60,103

switchport trunk encapsulation dot1q

switchport mode trunk

channel-group 34 mode active

!

interface GigabitEthernet0/2

switchport trunk allowed vlan 40,50,60,103

switchport trunk encapsulation dot1q

switchport mode trunk

channel-group 34 mode active

!

interface Vlan1

no ip address

shutdown

!

interface Vlan40

mac-address 0030.f244.8301

ip address 10.0.0.195 255.255.255.192

standby 40 ip 10.0.0.193

standby 0 preempt

!

interface Vlan50

mac-address 0030.f244.8302

ip address 10.0.1.3 255.255.255.192

standby 50 ip 10.0.1.1

standby 50 preempt

!

interface Vlan60

mac-address 0030.f244.8303

ip address 10.0.1.67 255.255.255.192

standby 60 ip 10.0.1.65

standby 60 preempt

!

interface Vlan103

mac-address 0030.f244.8304

ip address 10.0.1.195 255.255.255.224

standby 103 ip 10.0.1.193

standby 103 preempt

!

router ospf 1

router-id 1.1.2.2

log-adjacency-changes

network 10.0.0.192 0.0.0.63 area 0

network 10.0.1.0 0.0.0.63 area 0

network 10.0.1.64 0.0.0.63 area 0

network 10.0.1.192 0.0.0.31 area 0

network 10.0.100.4 0.0.0.3 area 0

!

ip classless

!

ip flow-export version 9

!

no cdp run

!

snmp-server community organisationX_RO RO

snmp-server community organisationX_RW RW

!

line con 0

!

line aux 0

!

line vty 0 4

login local

line vty 5 15

login local

!

end

Листинг 7. Running-Config маршрутизатора RA

Current configuration : 1151 bytes

!

version 15.1

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname RouterA

!

ip cef

no ipv6 cef

!

username admin privilege 15 secret 5 $1$mERr$RldxcCZEZsTFTETUyRaA50

!

license udi pid CISCO2911/K9 sn FTX1524HEZI-

!

ip ssh version 1

ip domain-name organisation.local

!

spanning-tree mode pvst

!

interface GigabitEthernet0/0

ip address 10.0.100.10 255.255.255.252

duplex auto

speed auto

!

interface GigabitEthernet0/1

ip address 10.0.100.22 255.255.255.252

duplex auto

speed auto

!

interface GigabitEthernet0/2

ip address 10.0.100.25 255.255.255.252

duplex auto

speed auto

!

interface Vlan1

no ip address

shutdown

!

router ospf 1

router-id 2.2.2.1

log-adjacency-changes

network 10.0.100.24 0.0.0.3 area 0

network 10.0.100.20 0.0.0.3 area 0

network 10.0.100.8 0.0.0.3 area 0

!

ip classless

!

ip flow-export version 9

!

no cdp run

!

snmp-server community organisationX_RO RO

snmp-server community organisationX_RW RW

!

line con 0

!

line aux 0

!

line vty 0 4

login local

line vty 5 15

login local

!

end

Листинг 8. Running-Config маршрутизатора RB

Current configuration : 1152 bytes

!

version 15.1

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname RouterB

!

ip cef

no ipv6 cef

!

username admin privilege 15 secret 5 $1$mERr$RldxcCZEZsTFTETUyRaA50

!

license udi pid CISCO2911/K9 sn FTX1524UH51-

!

ip ssh version 1

ip domain-name organisation.local

!

spanning-tree mode pvst

!

interface GigabitEthernet0/0

ip address 10.0.100.14 255.255.255.252

duplex auto

speed auto

!

interface GigabitEthernet0/1

ip address 10.0.100.18 255.255.255.252

duplex auto

speed auto

!

interface GigabitEthernet0/2

ip address 10.0.100.29 255.255.255.252

duplex auto

speed auto

!

interface Vlan1

no ip address

shutdown

!

router ospf 1

router-id 2.2.2.2

log-adjacency-changes

network 10.0.100.28 0.0.0.3 area 0

network 10.0.100.16 0.0.0.3 area 0

network 10.0.100.12 0.0.0.3 area 0

!

ip classless

!

ip flow-export version 9

!

no cdp run

!

snmp-server community organisationX_RO RO

snmp-server community organisationX_RW RW

!

line con 0

!

line aux 0

!

line vty 0 4

login local

line vty 5 15

login local

!

end